Nikhil

BCSE3 A2

ROLL 001810501069

ASSIGNMENT 3

TITLE

In this assignment, you have to implement p-persistent CSMA with exponential backoff and

additive backoff. Measure the performance parameters like throughput (i.e., average amount

of data bits successfully transmitted per unit time) and forwarding delay (i.e., average end-to--

end delay, including the queuing delay and the transmission delay) experienced by the CSMA

frames (IEEE 802.3). Plot the comparison graphs for throughput and forwarding delay by

varying p. State your observations on the impact of different data rates for exponential/additive

backoff along with p-persistent CSMA

DESIGN

The main purpose of the assignment to understand and then implement the working multiple Access protocols which are used in networking.

My design of is as follow :

- Channel Class – It represent the type of scheme we are using to simulate the process. Hence there are channel classes named –

- OONECHANNEL to simulate the ONEPERSISTENT MODEL.

- PCHANNEL to simulate the PPERISTENT MODEL.

- NPCHANNEL to simulate the NON PERSISTENT MODEL.

- Persistent Classes – It represent a particular type of station in the system. Hence they are also 3 in number.

- ONEPERSISTENT this station will use ONEPERSISTENT technique to send the data.

- NONPERSISTENT this station will use NONPERSISTENT technique to send the data.

- PPERSISTENT this station will use PPERSISTENT technique to send the data.

Here we have assumed that we have 5  station which will be sending data through the channel. With the logic we have implemented.

Each station will try to send the frame. But before sending the frame. Every station will sense the channel for its availability. Now Since we have 3 technique  next step depends on the mechanism used.

In case of

One Persistent – If channel is free. It will send it immediately. Otherwise it will start sensing the channel. Till the time channel gets free and when channel is free it will send the frame

Station 5

Station 4

Station 3

Station 2

Station 1

Main

Thread 1

Thread 4

Thread 5

Thread 2

Thread 3

IMPLEMENTATION/Code

One Persistent

package onepersistent;

import utilclasses.ChannelConstants;

import java.io.FileWriter;
import java.io.IOException;
import java.util.Random;
import java.util.concurrent.atomic.*;

class OneChannel implements Runnable, ChannelConstants {
    String stationNumber;
    Thread t;
    static int distance;
    static int stat=0;
    static int frame;
    static int channelStatus; //Indicates if channel is being used
    int currentFrameNumber;
    int maxFrameNumber;
    private final AtomicBoolean checkIfSuccessfulTransmission;
    static int tfr=50; //Transmission time
    private int numberOfAttempts;
    Random rand ;
    private static final  Object distanceLock = new Object();
    private static final Object channelUpdateLock = new Object();
    private static final Object getStatLock = new Object();
    FileWriter fileWriter ;

    OneChannel(String threadName, int maxFrameNumber, FileWriter fileWriter) {
        stationNumber = threadName;
        t = new Thread(this, stationNumber);
        currentFrameNumber = 1;
        this.maxFrameNumber =maxFrameNumber;
        checkIfSuccessfulTransmission = new AtomicBoolean();
        this.fileWriter = fileWriter;
        t.start();
    }
    public void run() {
        rand = new Random();
        while (!checkIfSuccessfulTransmission.get()) {
            numberOfAttempts++;
            while(currentFrameNumber <= maxFrameNumber) {
                OnePersistent.updateTotalCount();
                if (numberOfAttempts < 15) { //15 is the maximum number of attempts
                    try {
                        if (channelStatus == INUSE) {
                            System.out.println(stationNumber + " is using 1 Persistent sensing, channel is busy");
                            synchronized (this){
                                try {
                                    wait(); //***Waits forever.***
                                }
                                catch (InterruptedException e) {
                                    System.out.println(("Interrupt"));
                                    Thread.currentThread().interrupt();
                                    System.exit(1);
                                }
                            }

                        }
                        else {
                            System.out.println(stationNumber + " is trying to transmit frame number : " + currentFrameNumber);

                            if (channelStatus == FREE && distance == 0) {//Successful transmission
                                synchronized (getStatLock){
                                    stat = OneChannelThreads.checking(Thread.currentThread().getName());
                                    if(stat == -1){
                                        System.out.println("Error Setting up stat.");
                                        System.exit(1);
                                    }
                                    frame = this.currentFrameNumber;
                                }
                                synchronized (channelUpdateLock){
                                    channelStatus = INUSE;
                                }
                                long start = System.nanoTime();
                                synchronized (distanceLock){
                                    distanceLock.wait(1000);
                                }
                                long end = System.nanoTime();
                                synchronized (this){
                                    OnePersistent.forwarding_delay += end-start;
                                }
                                OnePersistent.updateDoneCount();
                                fileWriter.write(stationNumber + " frame " + currentFrameNumber + " is successful.\n");
                                fileWriter.flush();
                                System.out.println(stationNumber + " frame " + currentFrameNumber + " is successful.");
                                checkIfSuccessfulTransmission.set(true);
                                currentFrameNumber++;
                                synchronized (distanceLock){
                                    distance = 0;
                                }
                                synchronized (channelUpdateLock){
                                    channelStatus = FREE;
                                }
                                synchronized (this){
                                    try{
                                        notifyAll(); /// ***Notify All doesnt works.***
                                    } catch (Exception e) {
                                        e.printStackTrace();
                                    }
                                }
                            }
                            else {//Collision has occurred
                                System.out.println("Collision for frame " + currentFrameNumber + " of " +
                                        stationNumber + " and frame " + frame + " of Station " + stat);

                                System.out.println("Retransmitting Station " + stat + "'s frame " + frame);
                                checkIfSuccessfulTransmission.set(false);
                                synchronized (channelUpdateLock){
                                    channelStatus = FREE;
                                }

                                numberOfAttempts++;
                                /*try {
                                    int anInt = rand.nextInt((int) (Math.pow(2,(double) numberOfAttempts - 1)));
                                    int backOffTime = anInt * tfr;
                                    Thread.sleep(backOffTime);

                                } catch (InterruptedException e) {
                                    System.out.println("Interrupted");
                                    Thread.currentThread().interrupt();
                                }*/
                            }
                        //    Thread.sleep(1000);


                        }
                    } catch (InterruptedException | IOException e) {
                        System.out.println(stationNumber + "Main Interrupted");
                        Thread.currentThread().interrupt();
                    }
                }
                else {
                    checkIfSuccessfulTransmission.set(true);
                    System.out.println("Too many attempts for frame " + currentFrameNumber + "of " +
                            stationNumber + ". Transmission stopped");
                }

            }
        }
    }
}

UTILITY

package onepersistent;

import utilclasses.ChannelConstants;

/*Class to check which station's frame the current transmission is colliding with.

 */
class OneChannelThreads implements ChannelConstants {
    public static int checking(String StationName)
    {   int stat;
        char c = StationName.charAt(8);
        stat = Integer.parseInt(c+"");
        return(stat);
    }
}

ONE CHANNEL

package onepersistent;

import utilclasses.ChannelConstants;

import java.io.FileWriter;
import java.io.IOException;
import java.util.Random;
import java.util.concurrent.atomic.*;

class OneChannel implements Runnable, ChannelConstants {
    String stationNumber;
    Thread t;
    static int distance;
    static int stat=0;
    static int frame;
    static int channelStatus; //Indicates if channel is being used
    int currentFrameNumber;
    int maxFrameNumber;
    private final AtomicBoolean checkIfSuccessfulTransmission;
    static int tfr=50; //Transmission time
    private int numberOfAttempts;
    Random rand ;
    private static final  Object distanceLock = new Object();
    private static final Object channelUpdateLock = new Object();
    private static final Object getStatLock = new Object();
    FileWriter fileWriter ;

    OneChannel(String threadName, int maxFrameNumber, FileWriter fileWriter) {
        stationNumber = threadName;
        t = new Thread(this, stationNumber);
        currentFrameNumber = 1;
        this.maxFrameNumber =maxFrameNumber;
        checkIfSuccessfulTransmission = new AtomicBoolean();
        this.fileWriter = fileWriter;
        t.start();
    }
    public void run() {
        rand = new Random();
        while (!checkIfSuccessfulTransmission.get()) {
            numberOfAttempts++;
            while(currentFrameNumber <= maxFrameNumber) {
                OnePersistent.updateTotalCount();
                if (numberOfAttempts < 15) { //15 is the maximum number of attempts
                    try {
                        if (channelStatus == INUSE) {
                            System.out.println(stationNumber + " is using 1 Persistent sensing, channel is busy");
                            synchronized (this){
                                try {
                                    wait(); //***Waits forever.***
                                }
                                catch (InterruptedException e) {
                                    System.out.println(("Interrupt"));
                                    Thread.currentThread().interrupt();
                                    System.exit(1);
                                }
                            }

                        }
                        else {
                            System.out.println(stationNumber + " is trying to transmit frame number : " + currentFrameNumber);

                            if (channelStatus == FREE && distance == 0) {//Successful transmission
                                synchronized (getStatLock){
                                    stat = OneChannelThreads.checking(Thread.currentThread().getName());
                                    if(stat == -1){
                                        System.out.println("Error Setting up stat.");
                                        System.exit(1);
                                    }
                                    frame = this.currentFrameNumber;
                                }
                                synchronized (channelUpdateLock){
                                    channelStatus = INUSE;
                                }
                                long start = System.nanoTime();
                                synchronized (distanceLock){
                                    distanceLock.wait(1000);
                                }
                                long end = System.nanoTime();
                                synchronized (this){
                                    OnePersistent.forwarding_delay += end-start;
                                }
                                OnePersistent.updateDoneCount();
                                fileWriter.write(stationNumber + " frame " + currentFrameNumber + " is successful.\n");
                                fileWriter.flush();
                                System.out.println(stationNumber + " frame " + currentFrameNumber + " is successful.");
                                checkIfSuccessfulTransmission.set(true);
                                currentFrameNumber++;
                                synchronized (distanceLock){
                                    distance = 0;
                                }
                                synchronized (channelUpdateLock){
                                    channelStatus = FREE;
                                }
                                synchronized (this){
                                    try{
                                        notifyAll(); /// ***Notify All doesnt works.***
                                    } catch (Exception e) {
                                        e.printStackTrace();
                                    }
                                }
                            }
                            else {//Collision has occurred
                                System.out.println("Collision for frame " + currentFrameNumber + " of " +
                                        stationNumber + " and frame " + frame + " of Station " + stat);

                                System.out.println("Retransmitting Station " + stat + "'s frame " + frame);
                                checkIfSuccessfulTransmission.set(false);
                                synchronized (channelUpdateLock){
                                    channelStatus = FREE;
                                }

                                numberOfAttempts++;
                                /*try {
                                    int anInt = rand.nextInt((int) (Math.pow(2,(double) numberOfAttempts - 1)));
                                    int backOffTime = anInt * tfr;
                                    Thread.sleep(backOffTime);

                                } catch (InterruptedException e) {
                                    System.out.println("Interrupted");
                                    Thread.currentThread().interrupt();
                                }*/
                            }
                        //    Thread.sleep(1000);


                        }
                    } catch (InterruptedException | IOException e) {
                        System.out.println(stationNumber + "Main Interrupted");
                        Thread.currentThread().interrupt();
                    }
                }
                else {
                    checkIfSuccessfulTransmission.set(true);
                    System.out.println("Too many attempts for frame " + currentFrameNumber + "of " +
                            stationNumber + ". Transmission stopped");
                }

            }
        }
    }
}

PPERSISTENT

package ppersistent;

import utilclasses.ChannelConstants;

import java.io.FileWriter;
import java.io.IOException;
import java.util.Random;
import java.util.concurrent.atomic.*;

class PChannel implements Runnable, ChannelConstants {
    String stationNumber;
    Thread t;
    static int distance;
    static int stat=0;
    static int frame;
    static int channelStatus; //Indicates if channel is being used
    int currentFrameNumber;
    int maxFrameNumber;
    private final AtomicBoolean checkIfSuccessfulTransmission;
    static int tfr=50; //Transmission time
    private int numberOfAttempts;
    Random rand ;
    private static final  Object distanceLock = new Object();
    private static final Object channelUpdateLock = new Object();
    private static final Object getStatLock = new Object();
    FileWriter fileWriter ;
    private static final double prob = 0.1;

    PChannel(String threadName, int maxFrameNumber, FileWriter fileWriter) {
        stationNumber = threadName;
        t = new Thread(this, stationNumber);
        currentFrameNumber = 1;
        this.maxFrameNumber =maxFrameNumber;
        checkIfSuccessfulTransmission = new AtomicBoolean();
        this.fileWriter = fileWriter;
        t.start();
    }
    public void run() {
        rand = new Random();
        while (!checkIfSuccessfulTransmission.get()) {
            numberOfAttempts++;
            while(currentFrameNumber <= maxFrameNumber) {
                PPersistent.updateTotalCount();
                if (numberOfAttempts < 15) { //15 is the maximum number of attempts
                    try {
                        if (channelStatus == INUSE) {
                            System.out.println(stationNumber + " is using 1 Persistent sensing, channel is busy");
                            synchronized (this){
                                try {
                                    wait(); //***Waits forever.***
                                }
                                catch (InterruptedException e) {
                                    System.out.println(("Interrupt"));
                                    Thread.currentThread().interrupt();
                                    System.exit(1);
                                }
                            }

                        }
                        else {
                            System.out.println(stationNumber + " is trying to transmit frame number : " + currentFrameNumber);

                            if (channelStatus == FREE && distance == 0) {//Successful transmission
                                if(rand.nextDouble() < prob){
                                    continue;
                                }
                                synchronized (getStatLock){
                                    stat = PChannelThreads.checking(Thread.currentThread().getName());
                                    if(stat == -1){
                                        System.out.println("Error Setting up stat.");
                                        System.exit(1);
                                    }
                                    frame = this.currentFrameNumber;
                                }
                                synchronized (channelUpdateLock){
                                    channelStatus = INUSE;
                                }
                                long start = System.nanoTime();
                                synchronized (distanceLock){
                                  /*  for (; distance < 9000000; distance++)
                                        for(int i =0;i<1000;i++){
                                            //simulate transmission over some distance
                                    }*/
                                    distanceLock.wait(1000);
                                    //Thread.sleep(1000);
                                }
                                long end = System.nanoTime();
                                synchronized (this){
                                    PPersistent.forwarding_delay += end-start;
                                }
                                PPersistent.updateDoneCount();
                                fileWriter.write(stationNumber + " frame " + currentFrameNumber + " is successful.\n");
                                fileWriter.flush();
                                System.out.println(stationNumber + " frame " + currentFrameNumber + " is successful.");
                                checkIfSuccessfulTransmission.set(true);
                                currentFrameNumber++;
                                synchronized (distanceLock){
                                    distance = 0;
                                }
                                synchronized (channelUpdateLock){
                                    channelStatus = FREE;
                                }
                                synchronized (this){
                                    try{
                                        notifyAll(); /// ***Notify All doesnt works.***
                                    } catch (Exception e) {
                                        e.printStackTrace();
                                    }
                                }
                            }
                            else {//Collision has occurred
                                System.out.println("Collision for frame " + currentFrameNumber + " of " +
                                        stationNumber + " and frame " + frame + " of Station " + stat);

                                System.out.println("Retransmitting Station " + stat + "'s frame " + frame);
                                checkIfSuccessfulTransmission.set(false);
                                synchronized (channelUpdateLock){
                                    channelStatus = FREE;
                                }

                                numberOfAttempts++;
                                /*
                                try {
                                    int anInt = rand.nextInt((int) (Math.pow(2,(double) numberOfAttempts - 1)));
                                    int backOffTime = anInt * tfr;
                                    Thread.sleep(backOffTime);

                                } catch (InterruptedException e) {
                                    System.out.println("Interrupted");
                                    Thread.currentThread().interrupt();
                                }*/
                            }
                        //    Thread.sleep(1000);


                        }
                    } catch (InterruptedException | IOException e) {
                        System.out.println(stationNumber + "Main Interrupted");
                        Thread.currentThread().interrupt();
                    }
                }
                else {
                    checkIfSuccessfulTransmission.set(true);
                    System.out.println("Too many attempts for frame " + currentFrameNumber + "of " +
                            stationNumber + ". Transmission stopped");
                }

            }
        }
    }
}

package ppersistent;

import utilclasses.ChannelConstants;

/*Class to check which station's frame the current transmission is colliding with.

 */
public class PChannelThreads implements ChannelConstants {
    public static int checking(String StationName)
    {   int stat;
        char c = StationName.charAt(8);
        stat = Integer.parseInt(c+"");
        return(stat);
    }
}

package ppersistent;

import utilclasses.ChannelConstants;
import java.io.FileWriter;
import java.io.IOException;
import java.util.Date;
import java.util.Scanner;

class PPersistent implements ChannelConstants {
    private  static int totalChannelAttempt = 0 ;
    private  static int totalDoneAttempt = 0 ;
    public static long forwarding_delay = 0 ;
    private static long FRAME_SIZE = 520 ;


    public static synchronized void  updateTotalCount(){
        totalChannelAttempt++;
    }
    public static  synchronized  void updateDoneCount(){
        totalDoneAttempt++;
    }
    public static void main(String[] args){
        Scanner sc = new Scanner(System.in);
        PChannel.channelStatus = FREE; //initially channel is free
        System.out.println("Enter number of stations");
        int numberOfStations = sc.nextInt();
        PChannel[] channels= new PChannel[numberOfStations+1];
        int[]  noOfFramesInChannel = new int[numberOfStations+1];
        for(int i = 1;i<=numberOfStations;i++)
        {
            System.out.println("Enter number of frames for Station " + i);
            noOfFramesInChannel[i] = sc.nextInt();

        }
        FileWriter fileWriter = null;
        try {
            fileWriter = new FileWriter("logs.txt",true);
            fileWriter.write("Started Frame writing at "+new Date()+"\n");
            fileWriter.flush();
        } catch (IOException e) {
            e.printStackTrace();
            System.exit(1);
        }
        long start = System.nanoTime();
        for(int i = 1;i<=numberOfStations;i++)
            channels[i] = new PChannel("Station "+i,noOfFramesInChannel[i],fileWriter);

        try {
            for(int i=1;i<=numberOfStations;i++)
                channels[i].t.join();
        }
        catch (InterruptedException e) {
            System.out.println("Main Thread Interrupted");
            Thread.currentThread().interrupt();
            System.exit(1);
        }
        long end = System.nanoTime();
        long totalExecution = end - start;
        int count = 0 ;
        for(int i = 1;  i <=numberOfStations; ++i){
            count += noOfFramesInChannel[i];
        }
        System.out.println("Time taken to send "+count+" frames of "+numberOfStations+" channel including wait and queue time is "+totalExecution+" NanoSeconds.");
        double elapsedTimeInSecond = (double) forwarding_delay/ 1_000_000_000;
        long noOfBytes= FRAME_SIZE * count;
        double bps = noOfBytes/elapsedTimeInSecond;
        System.out.println("Total no. of bytes sent are "+noOfBytes+" and total time taken in sending the bytes is "+elapsedTimeInSecond+"sec ("+forwarding_delay+"ns)");
        System.out.println("Hence speed of the system is "+bps+" Bytes/sec.");
        double generated = totalChannelAttempt/elapsedTimeInSecond;
        double sent = totalDoneAttempt/elapsedTimeInSecond;
        System.out.println("No of frames generated in unit time are  "+generated);
        System.out.println("Successfully sent among the total generated are "+sent);
        System.out.println("Efficiency of the system is "+((sent/generated)*100));
        try {
            fileWriter.close();
        } catch (IOException e) {
            e.printStackTrace();
        }
        System.out.println("Transmission completed.");
    }
}

NON PERSISTENT

package nonpersistent;

import utilclasses.ChannelConstants;
import java.io.FileWriter;
import java.io.IOException;
import java.util.Date;
import java.util.Scanner;

class NonPersistent implements ChannelConstants {
    private  static int totalChannelAttempt = 0 ;
    private  static int totalDoneAttempt = 0 ;
    public static long forwarding_delay = 0 ;
    private static long FRAME_SIZE = 520 ;


    public static synchronized void  updateTotalCount(){
        totalChannelAttempt++;
    }
    public static  synchronized  void updateDoneCount(){
        totalDoneAttempt++;
    }
    public static void main(String[] args){
        Scanner sc = new Scanner(System.in);
        NPChannel.channelStatus = FREE; //initially channel is free
        System.out.println("Enter number of stations");
        int numberOfStations = sc.nextInt();
        NPChannel[] npChannels = new NPChannel[numberOfStations+1];
        int[]  noOfFramesInChannel = new int[numberOfStations+1];
        for(int i = 1;i<=numberOfStations;i++)
        {
            System.out.println("Enter number of frames for Station " + i);
            noOfFramesInChannel[i] = sc.nextInt();

        }
        FileWriter fileWriter = null;
        try {
            fileWriter = new FileWriter("logs.txt",true);
            fileWriter.write("Started Frame writing at "+new Date()+"\n");
            fileWriter.flush();
        } catch (IOException e) {
            e.printStackTrace();
            System.exit(1);
        }
        long start = System.nanoTime();
        for(int i = 1;i<=numberOfStations;i++)
            npChannels[i] = new NPChannel("Station "+i,noOfFramesInChannel[i],fileWriter);

        try{
           for(int i=1;i<=numberOfStations;i++)
                npChannels[i].t.join();
        } catch (InterruptedException e) {
            System.out.println("Main Thread Interrupted");
            Thread.currentThread().interrupt();
            System.exit(1);
        }
        long end = System.nanoTime();
        long totalExecution = end - start;
        int count = 0 ;
        for(int i = 0 ;  i < numberOfStations; ++i){
            count += noOfFramesInChannel[i];
        }
        System.out.println("Time taken to send "+count+" frames of "+numberOfStations+" channel including wait and queue time is "+totalExecution+" NanoSeconds.");
        double elapsedTimeInSecond = (double) forwarding_delay/ 1_000_000_000;
        long noOfBytes= FRAME_SIZE * count;
        double bps = noOfBytes/elapsedTimeInSecond;
        System.out.println("Total no. of bytes sent are "+noOfBytes+" and total time taken in sending the bytes is "+elapsedTimeInSecond+"sec ("+forwarding_delay+"ns)");
        System.out.println("Hence speed of the system is "+bps+" Bytes/sec.");
        double generated = totalChannelAttempt/elapsedTimeInSecond;
        double sent = totalDoneAttempt/elapsedTimeInSecond;
        System.out.println("No of frames generated in unit time are  "+generated);
        System.out.println("Successfully sent among the total generated are "+sent);
        System.out.println("Efficiency of the system is "+((sent/generated)*100));
        try {
            fileWriter.close();
        } catch (IOException e) {
            e.printStackTrace();
        }
        System.out.println("Transmission completed.");
    }
}

package nonpersistent;

import utilclasses.ChannelConstants;

import java.io.FileWriter;
import java.io.IOException;
import java.util.Random;
import java.util.concurrent.atomic.*;

class NPChannel implements Runnable, ChannelConstants {
    String stationNumber;
    Thread t;
    static int distance;
    static int stat=0;
    static int frame;
    static int channelStatus; //Indicates if channel is being used
    int currentFrameNumber;
    int totalNoOfFrames;
    private final AtomicBoolean checkIfSuccessfulTransmission;
    static int tfr=50; //Transmission time
    private int numberOfAttempts;
    Random rand ;
    private static final  Object distanceLock = new Object();
    private static final Object channelUpdateLock = new Object();
    private static final Object getStatLock = new Object();
    FileWriter fileWriter ;

    NPChannel(String threadName, int totalNoOfFrames, FileWriter fileWriter) {
        stationNumber = threadName;
        t = new Thread(this, stationNumber);
        currentFrameNumber = 1;
        this.totalNoOfFrames =totalNoOfFrames;
        checkIfSuccessfulTransmission = new AtomicBoolean();
        this.fileWriter = fileWriter;
        t.start();
    }
    public void run() {
        rand = new Random();
        while (!checkIfSuccessfulTransmission.get()){
            numberOfAttempts++;
            while(currentFrameNumber <= totalNoOfFrames) {
                NonPersistent.updateTotalCount();
                if (numberOfAttempts < 15) { //15 is the maximum number of attempts
                    try {
                        if (channelStatus == INUSE) {
                            System.out.println(stationNumber + " is using Non-Persistent sensing, channel is busy");
                            try {
                                long time =(long) rand.nextInt(50)+1000;
                                Thread.sleep(time);
                            }
                            catch (InterruptedException e) {
                                System.out.println(("Interrupt"));
                                Thread.currentThread().interrupt();
                            }
                        }
                        else {
                            System.out.println(stationNumber + " is trying to transmit frame number : " + currentFrameNumber);

                            if (channelStatus == FREE && distance == 0) {//Successful transmission
                                synchronized (distanceLock){
                                    distance++;
                                }
                                synchronized (getStatLock){
                                    stat = NPChannelThreads.checking(Thread.currentThread().getName());
                                    if(stat == -1){
                                        System.out.println("Error Setting up stat.");
                                        System.exit(1);
                                    }

                                    frame = this.currentFrameNumber;
                                }
                                synchronized (channelUpdateLock){
                                    channelStatus = INUSE;
                                }
                                long start = System.nanoTime();
                                synchronized (distanceLock){
                                    distanceLock.wait(1000);
                                }
                                long end = System.nanoTime();
                                synchronized (this){
                                    NonPersistent.forwarding_delay += end-start;
                                }
                                NonPersistent.updateDoneCount();
                                fileWriter.write(stationNumber + " frame " + currentFrameNumber + " is successful.\n");
                                fileWriter.flush();
                                System.out.println(stationNumber + " frame " + currentFrameNumber + " is successful.");

                                checkIfSuccessfulTransmission.set(true);
                                currentFrameNumber++;
                                synchronized (distanceLock){
                                    distance = 0;
                                }
                                synchronized (channelUpdateLock){
                                    channelStatus = FREE;
                                }
                            }
                            else {//Collision has occurred
                                System.out.println("Collision for frame " + currentFrameNumber + " of " +
                                        stationNumber + " and frame " + frame + " of Station " + stat);

                                System.out.println("Retransmitting Station " + stat + "'s frame " + frame);
                                checkIfSuccessfulTransmission.set(false);
                                synchronized (channelUpdateLock){
                                    channelStatus = FREE;
                                }
                                numberOfAttempts++;
                                try {
                                    int anInt = rand.nextInt((int) (Math.pow(2,(double) numberOfAttempts - 1)));
                                    int backOffTime = anInt * tfr;
                                    Thread.sleep(backOffTime);

                                } catch (InterruptedException e) {
                                    System.out.println("Interrupted");
                                    Thread.currentThread().interrupt();
                                }
                            }
                            Thread.sleep(1000);


                        }
                    } catch (InterruptedException | IOException e) {
                        System.out.println(stationNumber + "Main Interrupted");
                        Thread.currentThread().interrupt();
                    }
                }
                else {
                    checkIfSuccessfulTransmission.set(true);
                    System.out.println("Too many attempts for frame " + currentFrameNumber + "of " +
                            stationNumber + ". Transmission stopped");
                }

            }
        }
    }
}

package nonpersistent;

/*Class to check which station's frame the current transmission is colliding with.

 */
public class NPChannelThreads {
    private NPChannelThreads(){throw new UnsupportedOperationException("Util Class.");}

    public static int checking(String name) {
        int stat = -1 ;
        try{
            stat = Integer.parseInt(name.substring(8));
        }catch (NumberFormatException e ){
            //Some Error.
            System.out.println("NPChannel name I got is "+name);
        }
        return  stat;
    }

}

TEST CASES

ONEPERSISTENT

P PERSISTENT (p =0.1)

P PERSISTENT (p =0.4)

NON PERSISTENT

RESULT

| Scheme | No. of Station (total no. of frames) | No. Of Collisions | Total Time Taken |
| --- | --- | --- | --- |
| One Persistent | 5(15) | 6 | 13 Seconds |
| Non Persistent | 5(15) | 12 | 15 Seconds |
| P – Persistent (p = 0.1) | 5(15) | 2 | 9 Seconds |
| P – Persistent (p = 0.4) | 5(15) | 5 | 11 Seconds |

Non Persistent

One Persistent

p Persistent(p = 0.1)

p Persistent(p = 0.4)

P-persistent:-

P=0.2

Run                 Throughput                   Forwarding delay

Run1                    252                                     2694

Run2                    338                                     2008

Run3                    230                                     2858

Average           271 bits/sec                  2500 millisec

P-persistent:-

P=0.4

Run                 Throughput                   Forwarding delay

Run1                    252                                     2694

Run2                    338                                     2008

Run3                    230                                     2858

Average           185 bits/sec                  3500 millisec

P-persistent:-

P=0.6

Run                 Throughput                   Forwarding delay

Run1                    123                                     5528

Run2                    129                                     5263

Run3                    120                                     5643

Average           120 bits/sec                  5400 millisec

Analysis

From the above table and graph we can draw the conclusion that P- persistent is a effective technique we can use to handle multiple access. But I found that this p-persistent depends darastically on the value of choose P. minor change in P can lead to great difference in performance.

Now Coming to the part how to choose the value of P ????

P must be set to a value such that :

- It is very likely that exactly one node will pick a random number x such that

  - x < p

i.e. exactly 1 node should transmit its message.

- Using probability  theory

  - If n nodes are sensing, then the probability:

P = 1/n

will maximize the likelihood that exactly one node will transmit and (n-1)

nodes will back off.

EXAMPLE

- Consider that 2 nodes are waiting for the current transmission to finish then

We should choose

p = 0.5

- Now consider that in some case 3 nodes are waiting for the current transmission to

finish , then we should use

p = 0.33333333333(1/3)

Similarly if we have 4 nodes waiting then we should choose

P = 0.25

And so on……………….

Conclusion

P must be dynamic in nature.

Since it’s hard to know how many notes are busy. So a careful analysis should be done while picking the p.